📘 ESP32 Air Quality Sensor Node (ESP-NOW Sender Only)

📌 Project Description

This ESP32 board:

- Reads CO₂ sensor (UART)

- Reads BME680 (Temperature, Humidity, Pressure, Gas)

- Sends sensor data wirelessly using ESP-NOW

- Sends data to a receiver ESP32 using its MAC address

🧰 1. Components Required

- ESP32 Dev Board

- CO₂ Sensor (UART type – MH-Z19 / SenseAir type)

- BME680 Sensor (I2C)

- 5V Battery or regulated power supply

- ON/OFF Switch

- Jumper wires

🔌 2. Wiring Connections

✅ A) CO₂ Sensor → ESP32 (UART2)

| CO₂ Sensor Wire | ESP32 Pin |
| --- | --- |
| Red (VIN) | VIN (5V) |
| Black (GND) | GND |
| Blue (RX) | GPIO33 (TX2) |
| Green (TX) | GPIO32 (RX2) |

Important:

- Sensor TX → ESP32 RX (GPIO32)

- Sensor RX → ESP32 TX (GPIO33)

- Baud Rate = 9600

✅ B) BME680 → ESP32 (I2C)

| BME680 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | GPIO23 |
| SCL | GPIO22 |

⚠️ BME680 must use 3.3V only

✅ C) Battery Connection

Battery Positive → Switch → VIN
Battery Negative → GND

All grounds must be common.

🚀 3. Upload the Given Code