Soil Monitoring Node using 7-in-1 NPK Sensor with ESP32 (ESP-NOW Sender)

📌 Project Description

This ESP32 node:

- Reads soil data from 7-in-1 NPK RS485 sensor

- Uses TTL-RS485 module

- Communicates using Modbus RTU

- Sends parsed values using ESP-NOW

- Broadcasts data wirelessly

🧰 1. Components Required

- ESP32 Dev Board

- 7-in-1 NPK Soil Sensor (RS485)

- TTL to RS485 Module (MAX485)

- 12V DC Power Adapter

- Jumper Wires

🔌 2. Wiring Connections

✅ A) NPK Sensor → RS485 Module

| NPK Wire | Connect To |
| --- | --- |
| A (Yellow) | A+ |
| B (Blue) | B- |
| V+ (Brown) | 12V DC + |
| GND (Black) | 12V DC – |

⚠ Important:

- Sensor requires 12V power

- A → A+

- B → B-

✅ B) RS485 Module → ESP32

| ESP32 Pin | RS485 Module |
| --- | --- |
| GPIO16 | RXD / RO |
| GPIO17 | TXD / DI |
| GND | GND |
| 3.3V or 5V | VCC |

Important:

- GPIO16 = RX2

- GPIO17 = TX2

- Common ground required

📡 3. Communication Details

- Protocol: Modbus RTU

- Baud Rate: 4800

- Data Format: 8N1

- Slave ID: 0x01

- Register Read: 7 parameters

📊 4. Parameters Read

| Parameter | Unit |
| --- | --- |
| Moisture | % |
| Temperature | °C |
| Conductivity | µS/cm |
| pH | pH |
| Nitrogen | mg/kg |
| Phosphorus | mg/kg |
| Potassium | mg/kg |

🚀 5. Upload the Given Code

Upload the following code to ESP32.