Soil Moisture Monitoring Node using XIAO ESP32-C3 with ESP-NOW

📌 Project Description

This system:

- Uses Capacitive Soil Moisture Sensor

- Connected to Seeed Studio XIAO ESP32-C3

- Reads analog soil value

- Sends value wirelessly using ESP-NOW

- Sends data to receiver ESP32 using MAC address

🧰 1. Components Required

- Seeed Studio XIAO ESP32-C3

- Capacitive Soil Moisture Sensor v1.2

- 3.7V Battery

- ON/OFF Switch

- Jumper wires

🔌 2. Wiring Connections

✅ Soil Moisture Sensor → XIAO ESP32-C3

| Sensor Pin | Connect To |
| --- | --- |
| VCC | 3V3 |
| GND | GND |
| OUT | D4 (GPIO4) |

✅ Battery Connection

Battery Positive → Switch → 5V Pin
Battery Negative → GND

⚠ Important:

- All grounds must be common

- Sensor must be powered from 3.3V

- DO NOT connect sensor VCC to 5V

📡 3. Working Principle

- Soil moisture sensor outputs analog voltage

- ESP32 reads analog value using analogRead()

- Value stored in structure

- Sent using ESP-NOW

- Repeats every 2 seconds

🚀 4. Upload given code